Basic Two-Router Network - Using Static Routing Topology

Overview

This lab demonstrates how two separate Local Area Networks (LANs) can communicate with each other through routers. Two Cisco ISR4331 routers are connected using a point-to-point network and each router connects to a switch that serves multiple hosts. The purpose of this lab is to configure IP addressing and static routing so that devices in both networks can successfully communicate.

### Network Topology

The network consists of two routers (R1 and R2), two switches (SW1 and SW2), and ten end devices. Five PCs are connected to SW1 in network 192.168.1.0/24 and five laptops are connected to SW2 in network 192.168.2.0/24. The routers are interconnected using network 10.10.10.0/24.

### IP Addressing Plan

Network 1 (192.168.1.0/24):

| Device | IP Address | Subnet Mask |
| --- | --- | --- |
| PC1 | 192.168.1.10 | 255.255.255.0 |
| PC2 | 192.168.1.11 | 255.255.255.0 |
| PC3 | 192.168.1.12 | 255.255.255.0 |
| PC4 | 192.168.1.13 | 255.255.255.0 |
| PC5 | 192.168.1.14 | 255.255.255.0 |
| R1 G0/0 | 192.168.1.1 | 255.255.255.0 |

Router-to-Router Network (10.10.10.0/30):

Password - cisco

| Device | Interface | IP Address | Subnet Mask |
| --- | --- | --- | --- |
| R1 | G0/1 | 10.10.10.1 | 255.255.255.252 |
| R2 | G0/1 | 10.10.10.2 | 255.255.255.252 |

Network 2 (192.168.2.0/24):

| Device | IP Address | Subnet Mask |
| --- | --- | --- |
| LT1 | 192.168.2.10 | 255.255.255.0 |
| LT2 | 192.168.2.11 | 255.255.255.0 |
| LT3 | 192.168.2.12 | 255.255.255.0 |
| LT4 | 192.168.2.13 | 255.255.255.0 |
| LT5 | 192.168.2.14 | 255.255.255.0 |
| R2 G0/0 | 192.168.2.1 | 255.255.255.0 |