# OSPF Multi‑Router Network Lab

## Overview

This project demonstrates the configuration of a multi‑router network using the Open Shortest Path First (OSPF) dynamic routing protocol in Cisco Packet Tracer. The topology contains six routers connected in a ring, with several LAN networks connected through switches and PCs. The goal of this lab is to allow all devices in different networks to communicate automatically using OSPF.

## Network Topology

The topology includes six Cisco routers (R1–R6). Each router connects to two neighboring routers using 10.10.x.0 networks and also connects to a local LAN where PCs are located. Switches are used to connect PCs to the routers.

Topology Diagram

## Devices

Routers (2911): R1, R2, R3, R4, R5, R6

Switches (2900): SW1, SW2, SW3, SW4, SW5, SW6

PCs: PC1, PC2, PC3, PC4, PC5, PC6

## LAN Networks

The following LAN networks are used in this project:

| Network | Devices |
| --- | --- |
| 192.168.1.0/27 | R1 |
| 192.168.1.32/27 | R2 |
| 192.168.1.64/27 | R3 |
| 192.168.1.96/27 | R4 |
| 192.168.1.128/27 | R5 |
| 192.168.1.160/27 | R6 |

## Router Interconnections

Routers communicate with each other using the following networks:

| Link | Network |
| --- | --- |
| R1 ↔ R2 | 10.0.0.0/30 |
| R2 ↔ R3 | 10.0.0.4/30 |
| R3 ↔ R4 | 10.0.0.8/30 |
| R4 ↔ R5 | 10.0.0.12/30 |
| R5 ↔ R6 | 10.0.0.16/30 |
| R6 ↔ R1 | 10.0.0.20/30 |

## OSPF Configuration

Each router advertises its connected networks so that all routers can dynamically learn routes to every LAN in the topology.

Next configuration commands are used in each routers:

To create OSPF

#router ospf 1 /2/3/4/5/6
#network 0.0.0.0 255.255.255.255 area 0

To create Loopback 0 interface

#interface loopback 0

#ip address 1.1.1.1 255.255.255.255

To make passie interfaces of LAN-ing interaces

#router ospf 1

#passive interface g0/0

## Verification

After configuring OSPF, connectivity between all PCs is verified using the ping command. The following commands are also used on routers to confirm proper operation:

#show ip ospf neighbor

#show ip route

#ping <destination IP>

## Results

This lab demonstrates how dynamic routing with OSPF which allows multiple routers to automatically exchange routing information and build a complete routing table.  The result is full connectivity between all networks in the topology.